INDIAN INSTITUTE OF TECHNOLOGY, KHARAGPUR

RELIABILITY TESTING LABORATORY

RE69003

Subir Chowdhury School of Quality and Reliability

Autumn (2025 – 2026)

LT CDR ARAVIND MANIKANDAN (25RE61D01)

KADIMI TARUN (25RE61R05)

SUTAMA SARKAR (25RE61R07)

ACKNOWLEDGEMENT

1.	We would like to express our deepest gratitude to the Head of the Department - Prof. Neeraj Kumar Goyal, whose expert guidance and insightful teachings in Reliability Engineering have been invaluable throughout this laboratory work. His continuous encouragement and rigorous academic inputs have significantly enhanced our understanding and skills on the subject. We are truly thankful for his mentorship and the time he devoted to helping us strengthen our analytical and practical abilities in this domain.

2.	We are also sincerely grateful to Prof. Sanjay Kumar Chaturvedi and Prof. Heeralal Gargama for their valuable insights through academic sessions on the subject “Reliability Estimation and Life Testing”. Their guidance have played a crucial role in shaping our learning experience.

3.	We extend our heartfelt gratitude to Mr. Tanmay Mondal, our lab-in-charge, for his consistent assistance, smooth coordination of lab activities, and technical support.

4.	Finally, we thank all our seniors and fellow students for their cooperation, collaborative learning spirit, and mutual support throughout the lab work, which made this journey both productive and enriching.

INDEX

| Para No | Paragraph Title | Page No |
| --- | --- | --- |
| 1 | Aim | 4 |
| 2 | Introduction | 4 |
| 3 | Applications of Electromechanical Relays | 4 |
| 4 | Construction | 4 |
| 5 | Specification of Relay Under Test | 5 |
| 6 | Operating Conditions | 5 |
| 7 | Experimental Setup | 5 |
| 8 | Experimental Procedure | 6 |
| 9 | Test Results | 8 |
| 10 | Distribution Fitting of the Failure Times using Least Square Estimation (LSE) Method | 10 |
| 11 | Determination of Acceleration Factor Graphically | 20 |
| 12 | Reliability & Failure Probability Curves at Normal Use Temperature | 23 |
| 13 | Inferences | 25 |
| 14 | Conclusion | 25 |
| 15 | Recommendations | 25 |

RELAY LAB EXPERIMENT

- Aim. To conduct accelerated life testing on relays to estimate reliability using different estimation methods.

- Introduction.

2.1.	Electromechanical relays consist of a coil, an armature, a set of contacts, and a spring.

2.2.	When an electrical current flows through the coil, it creates a magnetic field that attracts the armature, causing the contacts to close or open.

2.3.	This mechanical action allows relays to control high-power circuits with a low-power input signal.

2.4.	In order to estimate the reliability of relays, a sample of 10 relays were selected for accelerated life testing at three different (130⁰C, 110⁰C and 90⁰C) temperatures.

2.5.	The time to failures of relays were recorded. Arrhenius model was selected as accelerated life model and data is analysed using Minitab software as well as manual calculation.

3.	Applications of Electromechanical Relays.	The following are some of the critical applications of Electromechanical Relays:-

### 3.1.	Power System Protection.	Used in overcurrent, overvoltage, undervoltage, distance, and earth fault protection.

### 3.2.	Control Systems & Automation.	Used in control panels for: Starter circuits, safety alarms.

### 3.3.	Home Appliances.    Refrigerators, washing machines, microwave ovens use relays for switching compressors, heaters, and motors.

### 3.4.	Timing & Sequential Control.	When combined with timers, relays create sequence control for: Packaging machines, Elevators.

4.	Construction.

4.1.	The contacts of the relays are metallic surfaces that open or close when the armature moves.

4.2.	These contacts are positioned to either complete or interrupt the electrical circuit in response to the movement of the armature.

4.3.	The relay has one common terminal and two contacts in two different configurations (normally open and normally closed).

4.4.	The relay switches between two circuits, when there is no voltage applied to the coil, one circuit is closed and the other one is open. When the coil is de-energized the opposite happens.

5.	Specification of Relay Under Test.

5.1.	Type: Electromechanical Power Relay

5.2.	Model: JQC-3FC (T73)

5.3.	Coil Type: DC Coil

5.4.	Rated Coil Voltage: 12 V DC

5.5.	Operating Frequency: 50/60 Hz (for AC switched loads)

Fig 1 – SPDT Relay

6.	Operating Condition.

  - Testing Profile: 1A 12V at 130ºC(403K), 120ºC (383K), 90ºC (363K).

6.2.	Switching: 4 cycles per minute (01 cycle = 5s ON & 10s OFF).

7.	Experimental Setup.

7.1.	The setup consists of 10 relays, 10 bulbs, 10 LEDs and 100 W load. 12V DC voltage is supplied to coil for excitation.

7.2.	The relays are kept in a temperature chamber for providing the test temperatures.

7.3.	Relays, bulbs and LEDs are connected as per the circuit diagram.

7.4.	Connecting wires are properly sieved with insulator to protect it from high temperature.

7.5.	The relay is considered fail when the designated bulb glows and failure time is recorded.

Fig 2 - Circuit Diagram

Fig 3 – Relay Main Circuit Board

8.	Experimental Procedure.

8.1.	Check and confirm all the equipment are properly connected as per the circuit diagram.

8.2.	Check all meters for calibration.

8.3.	Three samples of Relays were randomly taken up for the test. Each sample comprised 10 Relays (all of same make, model and batch). Relays are kept in the temperature chamber for exposure to test temperatures.

Fig 4 – Preparation of Test Set Up

8.4.	Switch on the temperature chamber. Check whether the test temperature is maintained properly in the test chamber.

Fig 5 – Thermal Chamber

8.5.	Switch on both main power supply and 12 V DC for excitation.

Fig 6 – Test Set Up – AC-DC Power Supply and Timer Circuit

8.6.	Note the reading of the counter when any of the relay experiences a failure.

9.	Test results.    Failures of Relays are observed at temperatures 130°C (403K), 120°C (393K), 90°C (363K) and the same are tabulated as below:-

9.1.	Test data at 130°C.

Table 7.1 – Test data at 130°C (403K)

| Relay Number | Failure times (cycles) | ti (accelerated condition) (hr) | Failure mode |
| --- | --- | --- | --- |
| R2 | 2724 | 11.350 | Short Circuit |
| R9 | 3324 | 13.850 |  |
| R6 | 3765 | 15.688 |  |
| R5 | 4457 | 18.571 |  |
| R10 | 5433 | 22.638 |  |
| R1 | 6088 | 25.367 |  |
| R3 | 6822 | 28.425 |  |
| R4 | 7699 | 32.079 |  |
| R8 | 8250 | 34.375 |  |
| R7 | 8542 | 35.592 |  |

9.2.	Test data at 120°C.

Table 7.2 – Test data at 120°C (393K)

| Relay Number | Failure times (cycles) | ti (accelerated condition) (hr) | Failure mode |
| --- | --- | --- | --- |
| R2 | 3022 | 12.59 | Short Circuit |
| R10 | 3627 | 15.11 |  |
| R4 | 4956 | 20.65 |  |
| R7 | 5416 | 22.57 |  |
| R3 | 6612 | 27.55 |  |
| R1 | 7210 | 30.04 |  |
| R5 | 8342 | 34.76 |  |
| R8 | 9076 | 37.82 |  |
| R6 | 10549 | 43.95 |  |
| R9 | 11351 | 47.30 |  |

9.3.	Test data at 90°C.

Table 7.3 – Test data at 90°C (363K)

| Relay Number | Failure times (cycles) | ti (accelerated condition) (hr) | Failure mode |
| --- | --- | --- | --- |
| R1 | 5568 | 23.20 | Short Circuit |
| R9 | 7123 | 29.68 |  |
| R4 | 8476 | 35.32 |  |
| R6 | 9920 | 41.33 |  |
| R8 | 11042 | 46.01 |  |
| R3 | 12457 | 51.90 |  |
| R2 | 13843 | 57.68 |  |
| R7 | 15321 | 63.84 |  |
| R5 | 16455 | 68.56 |  |
| R10 | 17863 | 74.43 |  |

10.	Distribution Fitting of the Failure Times using Least Square Estimation (LSE) Method.	LSE of the three failure data set was carried out through MS Excel and the relevant plots are attached below:-

10.1.	LSE of Failures Observed at 403K.	It can be observed from the below figures that the coefficient of correlation is higher and closest to 1 for Weibull distribution. This indicates that the failures observed at 403K can be best described through Weibull distribution.

Fig 7 – Probability Plots for Failure (cycles) Data iro 403K

10.2.	LSE of Failures Observed at 393K.	It can be observed from the below figures that the coefficient of correlation is higher and closest to 1 for Weibull distribution. This indicates that the failures observed at 393K can be best described through Weibull distribution.

Fig 8 – Probability Plots for Failure (cycles) Data iro 393K

10.3.	LSE of Failures Observed at 363K.	It can be observed from the below figures that the coefficient of correlation is higher and closest to 1 for Weibull distribution. This indicates that the failures observed at 363K can be best described through Weibull distribution.

Fig 9 – Probability Plots for Failure (cycles) Data iro 363K

10.4.	Weibull Distribution Parameters for Failures at different Temperatures.

Table 7.4 – Shape and Scale Parameter for Test Temperatures

| Temperature (in K) | Shape Parameter | Scale Parameter (in cycles) |
| --- | --- | --- |
| 403 | 2.734 | 6456.62 |
| 393 | 2.499 | 7967.12 |
| 363 | 2.917 | 13289.61 |

10.5.	F(ti) vs ti (in hrs) Curve.	With the values of parameters as determined above, F(ti) values for different values of time, are calculated and Failure CDF curves for different Temperatures are generated as below:-

Table 7.5 – F(ti) for Test Temperatures

| ti (in hrs) | F(ti) - 403K | F(ti) - 393K | F(ti) - 363K |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 1 | 0.000123269 | 0.00015803 | 8.2212E-06 |
| 2 | 0.00081982 | 0.000893005 | 6.2091E-05 |
| 3 | 0.002481937 | 0.002457909 | 0.00020261 |
| 4 | 0.005441623 | 0.005037624 | 0.00046886 |
| 5 | 0.009992766 | 0.008781851 | 0.00089875 |
| 6 | 0.016396877 | 0.013815183 | 0.00152923 |
| 7 | 0.024883898 | 0.020241426 | 0.00239644 |
| 8 | 0.035650838 | 0.028145646 | 0.00353575 |
| 9 | 0.048859332 | 0.037595123 | 0.00498171 |
| 10 | 0.064632743 | 0.048639767 | 0.00676804 |
| 11 | 0.08305321 | 0.061312262 | 0.00892757 |
| 12 | 0.104158958 | 0.075628103 | 0.01149214 |
| 13 | 0.127942132 | 0.091585639 | 0.0144925 |
| 14 | 0.154347371 | 0.109166175 | 0.01795822 |
| 15 | 0.183271327 | 0.128334192 | 0.02191753 |
| 16 | 0.214563259 | 0.149037723 | 0.02639725 |
| 17 | 0.248026813 | 0.171208898 | 0.03142259 |
| 18 | 0.283423048 | 0.194764678 | 0.03701705 |
| 19 | 0.320474697 | 0.219607783 | 0.04320226 |
| 20 | 0.358871613 | 0.245627816 | 0.04999784 |
| 21 | 0.398277285 | 0.272702571 | 0.05742125 |
| 22 | 0.438336242 | 0.300699514 | 0.06548764 |
| 23 | 0.478682128 | 0.329477419 | 0.07420973 |
| 24 | 0.518946196 | 0.358888129 | 0.08359765 |
| 25 | 0.558765901 | 0.388778428 | 0.09365882 |
| 26 | 0.597793307 | 0.418991975 | 0.10439787 |
| 27 | 0.63570299 | 0.449371277 | 0.11581646 |
| 28 | 0.67219914 | 0.479759662 | 0.12791327 |
| 29 | 0.707021614 | 0.510003221 | 0.14068384 |
| 30 | 0.739950717 | 0.539952672 | 0.15412058 |
| 31 | 0.770810563 | 0.56946512 | 0.16821268 |
| 32 | 0.799470933 | 0.598405684 | 0.18294611 |
| 33 | 0.825847606 | 0.62664895 | 0.19830361 |
| 34 | 0.849901227 | 0.654080241 | 0.21426472 |
| 35 | 0.871634828 | 0.680596664 | 0.23080579 |
| 36 | 0.891090189 | 0.706107949 | 0.24790008 |
| 37 | 0.908343252 | 0.730537031 | 0.26551786 |
| 38 | 0.923498848 | 0.753820415 | 0.28362649 |
| 39 | 0.93668501 | 0.775908285 | 0.30219057 |
| 40 | 0.948047122 | 0.796764402 | 0.32117215 |
| 41 | 0.957742179 | 0.816365771 | 0.34053085 |
| 42 | 0.965933351 | 0.834702122 | 0.36022412 |
| 43 | 0.97278506 | 0.851775202 | 0.38020744 |
| 44 | 0.97845868 | 0.867597925 | 0.40043463 |
| 45 | 0.983108979 | 0.882193386 | 0.42085802 |
| 46 | 0.986881316 | 0.895593785 | 0.44142886 |
| 47 | 0.989909611 | 0.907839276 | 0.46209751 |
| 48 | 0.992315038 | 0.918976778 | 0.48281381 |
| 49 | 0.994205378 | 0.929058773 | 0.50352738 |
| 50 | 0.995674946 | 0.938142112 | 0.52418796 |
| 51 | 0.996804968 | 0.946286862 | 0.5447457 |
| 52 | 0.997664336 | 0.953555209 | 0.56515151 |
| 53 | 0.998310603 | 0.960010428 | 0.58535736 |
| 54 | 0.998791145 | 0.965715951 | 0.6053166 |
| 55 | 0.999144394 | 0.970734524 | 0.62498426 |
| 56 | 0.999401085 | 0.975127473 | 0.64431729 |
| 57 | 0.999585442 | 0.978954081 | 0.6632749 |
| 58 | 0.999716293 | 0.982271068 | 0.68181872 |
| 59 | 0.999808065 | 0.985132188 | 0.69991308 |
| 60 | 0.999871655 | 0.987587917 | 0.71752517 |
| 61 | 0.999915184 | 0.989685247 | 0.73462522 |
| 62 | 0.999944615 | 0.991467563 | 0.75118662 |
| 63 | 0.999964268 | 0.992974598 | 0.76718606 |
| 64 | 0.999977228 | 0.99424246 | 0.78260356 |
| 65 | 0.999985666 | 0.995303716 | 0.79742256 |
| 66 | 0.999991089 | 0.99618752 | 0.81162989 |
| 67 | 0.99999453 | 0.996919788 | 0.82521578 |
| 68 | 0.999996685 | 0.997523388 | 0.8381738 |
| 69 | 0.999998017 | 0.998018364 | 0.85050077 |
| 70 | 0.999998829 | 0.998422159 | 0.86219668 |
| 71 | 0.999999318 | 0.998749854 | 0.87326455 |
| 72 | 0.999999608 | 0.9990144 | 0.88371026 |
| 73 | 0.999999777 | 0.999226842 | 0.89354239 |
| 74 | 0.999999875 | 0.99939654 | 0.90277203 |
| 75 | 0.999999931 | 0.999531374 | 0.91141257 |
| 76 | 0.999999963 | 0.999637934 | 0.91947948 |
| 77 | 0.99999998 | 0.999721698 | 0.92699009 |
| 78 | 0.999999989 | 0.999787186 | 0.93396335 |
| 79 | 0.999999994 | 0.999838109 | 0.94041959 |
| 80 | 0.999999997 | 0.999877491 | 0.94638029 |
| 81 | 0.999999999 | 0.99990778 | 0.95186785 |
| 82 | 0.999999999 | 0.999930948 | 0.95690538 |
| 83 | 1 | 0.99994857 | 0.96151644 |
| 84 | 1 | 0.999961901 | 0.96572488 |
| 85 | 1 | 0.999971928 | 0.96955459 |
| 86 | 1 | 0.999979428 | 0.97302938 |
| 87 | 1 | 0.999985007 | 0.97617277 |
| 88 | 1 | 0.999989133 | 0.97900783 |
| 89 | 1 | 0.999992166 | 0.98155709 |
| 90 | 1 | 0.999994385 | 0.98384238 |
| 91 | 1 | 0.999995997 | 0.98588476 |
| 92 | 1 | 0.999997163 | 0.9877044 |
| 93 | 1 | 0.999998 | 0.98932054 |
| 94 | 1 | 0.999998598 | 0.99075144 |
| 95 | 1 | 0.999999023 | 0.99201432 |
| 96 | 1 | 0.999999323 | 0.99312536 |
| 97 | 1 | 0.999999534 | 0.99409968 |
| 98 | 1 | 0.999999681 | 0.99495134 |
| 99 | 1 | 0.999999783 | 0.99569335 |
| 100 | 1 | 0.999999853 | 0.9963377 |

Fig 10 – Failure CDF Plot at different Test Temperatures

10.6.	R(ti) vs ti Curve.	With the values of parameters as determined above, R(ti) values for different values of time, are calculated and Reliability curves for different Temperature are generated as below:-

Table 7.6 – R(ti) for Test Temperatures

| ti (in hrs) | R(ti) - 403K | R(ti) - 393K | R(ti) - 363K |
| --- | --- | --- | --- |
| 0 | 1 | 1 | 1 |
| 1 | 0.99987673 | 0.99984197 | 0.99999178 |
| 2 | 0.99918018 | 0.99910699 | 0.99993791 |
| 3 | 0.99751806 | 0.99754209 | 0.99979739 |
| 4 | 0.99455838 | 0.99496238 | 0.99953114 |
| 5 | 0.99000723 | 0.99121815 | 0.99910125 |
| 6 | 0.98360312 | 0.98618482 | 0.99847077 |
| 7 | 0.9751161 | 0.97975857 | 0.99760356 |
| 8 | 0.96434916 | 0.97185435 | 0.99646425 |
| 9 | 0.95114067 | 0.96240488 | 0.99501829 |
| 10 | 0.93536726 | 0.95136023 | 0.99323196 |
| 11 | 0.91694679 | 0.93868774 | 0.99107243 |
| 12 | 0.89584104 | 0.9243719 | 0.98850786 |
| 13 | 0.87205787 | 0.90841436 | 0.9855075 |
| 14 | 0.84565263 | 0.89083383 | 0.98204178 |
| 15 | 0.81672867 | 0.87166581 | 0.97808247 |
| 16 | 0.78543674 | 0.85096228 | 0.97360275 |
| 17 | 0.75197319 | 0.8287911 | 0.96857741 |
| 18 | 0.71657695 | 0.80523532 | 0.96298295 |
| 19 | 0.6795253 | 0.78039222 | 0.95679774 |
| 20 | 0.64112839 | 0.75437218 | 0.95000216 |
| 21 | 0.60172271 | 0.72729743 | 0.94257875 |
| 22 | 0.56166376 | 0.69930049 | 0.93451236 |
| 23 | 0.52131787 | 0.67052258 | 0.92579027 |
| 24 | 0.4810538 | 0.64111187 | 0.91640235 |
| 25 | 0.4412341 | 0.61122157 | 0.90634118 |
| 26 | 0.40220669 | 0.58100802 | 0.89560213 |
| 27 | 0.36429701 | 0.55062872 | 0.88418354 |
| 28 | 0.32780086 | 0.52024034 | 0.87208673 |
| 29 | 0.29297839 | 0.48999678 | 0.85931616 |
| 30 | 0.26004928 | 0.46004733 | 0.84587942 |
| 31 | 0.22918944 | 0.43053488 | 0.83178732 |
| 32 | 0.20052907 | 0.40159432 | 0.81705389 |
| 33 | 0.17415239 | 0.37335105 | 0.80169639 |
| 34 | 0.15009877 | 0.34591976 | 0.78573528 |
| 35 | 0.12836517 | 0.31940334 | 0.76919421 |
| 36 | 0.10890981 | 0.29389205 | 0.75209992 |
| 37 | 0.09165675 | 0.26946297 | 0.73448214 |
| 38 | 0.07650115 | 0.24617959 | 0.71637351 |
| 39 | 0.06331499 | 0.22409171 | 0.69780943 |
| 40 | 0.05195288 | 0.2032356 | 0.67882785 |
| 41 | 0.04225782 | 0.18363423 | 0.65946915 |
| 42 | 0.03406665 | 0.16529788 | 0.63977588 |
| 43 | 0.02721494 | 0.1482248 | 0.61979256 |
| 44 | 0.02154132 | 0.13240208 | 0.59956537 |
| 45 | 0.01689102 | 0.11780661 | 0.57914198 |
| 46 | 0.01311868 | 0.10440622 | 0.55857114 |
| 47 | 0.01009039 | 0.09216072 | 0.53790249 |
| 48 | 0.00768496 | 0.08102322 | 0.51718619 |
| 49 | 0.00579462 | 0.07094123 | 0.49647262 |
| 50 | 0.00432505 | 0.06185789 | 0.47581204 |
| 51 | 0.00319503 | 0.05371314 | 0.4552543 |
| 52 | 0.00233566 | 0.04644479 | 0.43484849 |
| 53 | 0.0016894 | 0.03998957 | 0.41464264 |
| 54 | 0.00120886 | 0.03428405 | 0.3946834 |
| 55 | 0.00085561 | 0.02926548 | 0.37501574 |
| 56 | 0.00059891 | 0.02487253 | 0.35568271 |
| 57 | 0.00041456 | 0.02104592 | 0.3367251 |
| 58 | 0.00028371 | 0.01772893 | 0.31818128 |
| 59 | 0.00019194 | 0.01486781 | 0.30008692 |
| 60 | 0.00012834 | 0.01241208 | 0.28247483 |
| 61 | 8.4816E-05 | 0.01031475 | 0.26537478 |
| 62 | 5.5385E-05 | 0.00853244 | 0.24881338 |
| 63 | 3.5732E-05 | 0.0070254 | 0.23281394 |
| 64 | 2.2772E-05 | 0.00575754 | 0.21739644 |
| 65 | 1.4334E-05 | 0.00469628 | 0.20257744 |
| 66 | 8.9109E-06 | 0.00381248 | 0.18837011 |
| 67 | 5.4697E-06 | 0.00308021 | 0.17478422 |
| 68 | 3.3148E-06 | 0.00247661 | 0.1618262 |
| 69 | 1.983E-06 | 0.00198164 | 0.14949923 |
| 70 | 1.1709E-06 | 0.00157784 | 0.13780332 |
| 71 | 6.823E-07 | 0.00125015 | 0.12673545 |
| 72 | 3.9231E-07 | 0.0009856 | 0.11628974 |
| 73 | 2.2255E-07 | 0.00077316 | 0.10645761 |
| 74 | 1.2454E-07 | 0.00060346 | 0.09722797 |
| 75 | 6.8739E-08 | 0.00046863 | 0.08858743 |
| 76 | 3.7417E-08 | 0.00036207 | 0.08052052 |
| 77 | 2.0083E-08 | 0.0002783 | 0.07300991 |
| 78 | 1.0628E-08 | 0.00021281 | 0.06603665 |
| 79 | 5.5444E-09 | 0.00016189 | 0.05958041 |
| 80 | 2.8509E-09 | 0.00012251 | 0.05361971 |
| 81 | 1.4447E-09 | 9.222E-05 | 0.04813215 |
| 82 | 7.2143E-10 | 6.9052E-05 | 0.04309462 |
| 83 | 3.5494E-10 | 5.143E-05 | 0.03848356 |
| 84 | 1.7204E-10 | 3.8099E-05 | 0.03427512 |
| 85 | 8.2134E-11 | 2.8072E-05 | 0.03044541 |
| 86 | 3.862E-11 | 2.0572E-05 | 0.02697062 |
| 87 | 1.7882E-11 | 1.4993E-05 | 0.02382723 |
| 88 | 8.1527E-12 | 1.0867E-05 | 0.02099217 |
| 89 | 3.6592E-12 | 7.8336E-06 | 0.01844291 |
| 90 | 1.6167E-12 | 5.6154E-06 | 0.01615762 |
| 91 | 7.0299E-13 | 4.0029E-06 | 0.01411524 |
| 92 | 3.0087E-13 | 2.8374E-06 | 0.0122956 |
| 93 | 1.2668E-13 | 1.9999E-06 | 0.01067946 |
| 94 | 5.2514E-14 | 1.4017E-06 | 0.00924856 |
| 95 | 2.1427E-14 | 9.7677E-07 | 0.00798568 |
| 96 | 8.5487E-15 | 6.7677E-07 | 0.00687464 |
| 97 | 3.3307E-15 | 4.6621E-07 | 0.00590032 |
| 98 | 0 | 3.193E-07 | 0.00504866 |
| 99 | 0 | 2.1742E-07 | 0.00430665 |
| 100 | 0 | 1.4718E-07 | 0.0036623 |

Fig 11 – Reliability Plot at different Test Temperatures

11.	Determination of Acceleration Factor Graphically.    Since the Shape Parameter values vary for each experiment, an average value of the same is assumed for further calculation. Subsequently, Acceleration Factors were calculated and normal use temperatures were determined.

11.1.	Averaging of Shape Parameter.

average = (403K + 393K + 363K)/3 = 2.717

11.2.	Calculation of revised Scale Parameter (in hrs) and Mean Time To Failure (in hrs) with Average Shape Parameter value.	Revised Scale Parameter is calculated using Maximum Likelihood Equation:-

The MTTF values are calculated using formula:-

The revised θ and MTTF values are tabulated below:-

Table 7.7 – Revised Parameter Values for Test Temperatures

| Temperature (in K) | Shape Parameter | Scale Parameter (in cycles) | Mean Time to Failure (in cycles) |
| --- | --- | --- | --- |
| 403 | 2.717 | 6260 | 5568 |
| 393 |  | 7818 | 6954 |
| 363 |  | 12824 | 11407 |

11.3.	Plotting of ln (θ) vs Stress (1/T).		The values obtained above are plotted in MS Excel to observe the Arrhenius model equation and values of A, B are calculated from the straight line equation as shown below.

Fig 12 – Arrhenius Plot of ln(θ) vs Stress (1/T)

ln(θ) = ln A + B/T

ln A = 2.4286

A = 11.34

B = Ea/K = 2555

Activation Energy = Ea = 2555 x 8.617 * 10-5 = 0.2201eV

As per the Original Equipment Manufacturer’s specifications, the subject SPDT Relay will operate upto 10 million operations (mechanical cycles) under normal usage.

11.4.	Acceleration Factors (AFs). AFs are calculated as below and the normal use life of Relays observed in above three experiments are as below:-

AFt→u = θ(use)/θ(test)

AF403-298 = 10000000 / 6260 = 1597.44

AF393-298 = 10000000 / 7818 = 1279.09

AF363-298 = 10000000 / 12824 = 779.79

11.5.	Test data at 130°C to normal use condition using AF = 1597.44.

Table 7.8 – Life at Normal Use Condition as Extrapolated from 130°C (403K)

| Relay Number | ti (accelerated condition) (cycles) | ti (normal use condition) (cycles) |
| --- | --- | --- |
| R2 | 2724 | 4351427 |
| R9 | 3324 | 5309891 |
| R6 | 3765 | 6014362 |
| R5 | 4457 | 7119790 |
| R10 | 5433 | 8678892 |
| R1 | 6088 | 9725215 |
| R3 | 6822 | 10897736 |
| R4 | 7699 | 12298691 |
| R8 | 8250 | 13178880 |
| R7 | 8542 | 13645332 |

11.6.	Test data at 120°C to normal use condition using AF = 1279.09.

Table 7.9 – Life at Normal Use Condition as Extrapolated from 120°C (393K)

| Relay Number | ti (accelerated condition) (cycles) | ti (normal use condition) (cycles) |
| --- | --- | --- |
| R2 | 3022 | 3865410 |
| R10 | 3627 | 4639259 |
| R4 | 4956 | 6339170 |
| R7 | 5416 | 6927551 |
| R3 | 6612 | 8457343 |
| R1 | 7210 | 9222238 |
| R5 | 8342 | 10670169 |
| R8 | 9076 | 11609021 |
| R6 | 10549 | 13493120 |
| R9 | 11351 | 14518951 |

11.7.	Test data at 90°C to normal use condition using AF = 779.79.

Table 7.10 – Life at Normal Use Condition as Extrapolated from 90°C (363K)

| Relay Number | ti (accelerated condition) (cycles) | ti (normal use condition) (cycles) |
| --- | --- | --- |
| R1 | 5568 | 4341871 |
| R9 | 7123 | 5554444 |
| R4 | 8476 | 6609500 |
| R6 | 9920 | 7735517 |
| R8 | 11042 | 8610441 |
| R3 | 12457 | 9713844 |
| R2 | 13843 | 10794633 |
| R7 | 15321 | 11947163 |
| R5 | 16455 | 12831444 |
| R10 | 17863 | 13929389 |

12.	Reliability & Failure Probability Curves at Normal Use Temperature.

12.1.	The normal use Cycles To Failures of 30 Relays (as determined above) are arranged in an ascending sequence and using Lease Square Estimation method, the distribution which fits this data the best is determined to be Normal Distribution. Fitting of the data in LSEs are as below.

Fig 13 – Probability Plots for Cumulative Failure (cycles) Data

13.	Inferences.

13.1.	The cumulative failure data follows a Normal distribution, confirming it as a suitable model for relay lifetime analysis.

13.2.	The mean value estimated from the Normal distribution LSE indicates a mean life of 8746333 cycles which is close to OEM established life of 10 million cycles .

13.3.	The scale parameter (θ) of accelerated life failures increases with decreasing temperature, demonstrating that relay life improves significantly at lower thermal stress.

13.4.	The Arrhenius plot of ln(θ) vs 1/T produced a clear linear trend, validating the use of Arrhenius modelling for thermal acceleration of relay failures.

14.	Conclusion.    The analysis provides a preliminary understanding of the reliability of the relays under high-temperature conditions and high switching cycles. Key findings include:

14.1.	Temperature & Switching Cycles Sensitivity. Elevated temperatures and switching cycles significantly impact relay life, as demonstrated by high acceleration factors, indicating that maintaining lower operational temperatures and low switching rate is crucial for optimal relay lifespan.

14.2.	The mean life of the subject Relays estimated at normal-use condition is approximately 8746333 cycles.

14.3.	Short-circuit failures were consistently observed, indicating contact wear as the dominant failure mechanisms under the accelerated stress profile.

14.4.	The study confirms that temperature, switching rate, and electrical load collectively govern relay life, highlighting the importance of proper derating and operating conditions in practical applications.

15.	Recommendations.	In view of the above, the following are recommended to obtain better test results for future studies:-

15.1.	The switching frequency may be adjusted to reflect realistic field conditions so that the observed degradation corresponds to genuine wear-out mechanisms rather than overstress failures.

15.2.	Accurate calibration and uniform temperature control of the thermal chamber must be ensured to avoid localized overheating and improve the consistency of the results.

15.3.	Additionally, larger sample sizes and wider temperature spacing may be considered to achieve more reliable Arrhenius fitting.